Intro to Electronics Task

Task 1:

- Identify the needs

What voltage and current do each part need?

- Load A : Re22-Relay at 9v needs voltage= 9 VDC and current= 40 mA

- Load B: HC-SR04 Ultrasonic Sensor voltage= 5 VDC and current= 15 mA

Is the battery voltage higher or lower than what is needed?

The battery voltage is 12.6 v which is higher than what load A needs (9v) and load B (5v).

Do we need protection against connecting the battery backwards?

Yes, we need protection to avoid damaging the loads in case of accidentally connecting the battery backwards. We can use a diode because it only allows the passage of current in one direction or MOSFET.

- Choose the components

Pick a regulator type (Buck converter or LDO) for each output, and explain why

For load A Re22-Relay at 9v: LDO MIC29302A can be used to reduce voltage from 12.6v to 9v as it is a small difference in voltage and wouldn’t produce too much heat.

For load B HC-SR04 Ultrasonic Sensor: Buck converter LM2596S should be used to reduce the voltage from 12.6v to 5v as it is a significant reduction and would produce too much heat if we attempt to use LDO.

Pick protection circuit if needed, and explain why that protection

I pick MTP30P05 P-Channel Power MOSFET Transistor Where I can connect the source with battery and grain with LDO and BUCK and gate with ground so that it allows passage of current if the positive of battery is connected with source and prevent it in opposite direction, it is better than a diode as it has less voltage drop across it.

- Check the datasheet

LDO MIC29302A

Input voltage range is from 3 to 16 v which covers the 3S Li-ion (12.6V) battery and output current is up to 3 A, and the relay only require 40 mA, so it has more than enough

Buck converter LM2596S

Input voltage ranges from 3 to 40 v which covers the battery and output current up to 2A which is enough for load B which only need 15 mA

MTP30P05 P-Channel Power MOSFET Transistor

It has a V drain-source =50 v which is much larger than the 12.6 v battery and passes current up to 40 A which is more than enough.

- Simulate in LTspice

Comment: I simulated a buck converter circuit and added a switch controlled by a pulse voltage source that turns on the switch 71.4% of the time giving approximately 9v from the 12.6 v provided by the voltage source at the resistance R1.

Task 2:

• Capacitors

C11

It is a passive electrical component that stores electric energy in an electric field and releases it when needed.

They help keep voltage at the same level, remove noise, and can charge and discharge energy.

• Resistors

R11

It is a passive electrical component that restricts the passage of electrical current in the circuit.

They limit current that goes into fragile components like leds and microchips, can be used as voltage dividers, sets the correct operating points for other components like transistors and much more.

• Inductors

L1

It is a passive electrical component that stores energy in a magnetic field when current flows through it.

They are mainly used for functions of filtering, oscillation, and delay on a PCB; they also help stabilize current and voltage.

• Voltage Regulators

U4

It regulates input voltage to produce a lower stable output voltage.

They are used to provide the correct voltage needed by other components in the circuit.

• Connectors

J8

It is a component that allows you to connect wires, cables, and other circuits together without permanent soldering.

They provide physical or electrical connections so that the power or electric signal can enter or leave the PCB board.

• Diodes

D3

It is a two terminal electrical component that allows the flow of electricity in one way only.

They are used in rectification where they can be used to turn AC current to DC, circuit protection to make sure batteries and other components are connected correctly, voltage regulation like in case of Zener diode, they can convert like in case of  LED or photodiodes and much more.